2 Marks Questions

- Give the difference between serial communication and parallel communication.

| Feature | Serial Communication | Parallel Communication |
| --- | --- | --- |
| Data Transmission | Sends 1 bit at a time | Sends multiple bits simultaneously |
| Speed | Slower (one bit per cycle) | Faster (multiple bits per cycle) |
| Cost & Complexity | Low cost, simple wiring | Higher cost, complex wiring |
| Distance | Suitable for long distances | Suitable for short distances |
| Example | USB, UART | Printer ports, old parallel buses |

- State the function of pull-up resistors in GPIO circuits.

Answer:
A pull-up resistor is used to maintain a GPIO pin at a defined HIGH logic level (Vcc) when no active signal is present.

Key Functions:

- Prevents floating (undefined) state of the input pin

- Ensures a stable default HIGH value

- Allows the pin to be safely pulled LOW when connected to ground (e.g., via a switch)

- Mention the role of timers in microcontrollers.

Answer:
Timers in microcontrollers are used to measure time intervals and generate precise delays.

Roles:

- Generate time delays

- Count events or pulses

- Produce PWM signals for control applications

- Support interrupt generation at specific time intervals

- Define a task in RTOS.

Answer:
A task in an RTOS (Real-Time Operating System) is an independent unit of execution (a program or thread) that performs a specific function and is managed by the scheduler.

Key Point:
Each task has its own priority, stack, and state (ready, running, waiting).

- Describe context switching.

Answer:
Context switching is the process of saving the state (context) of a currently running task and restoring the state of another task so that multiple tasks can share the CPU.

Key Point:
The context includes CPU registers, program counter, and stack information.

- Outline the function of a tick timer.

Answer:
A tick timer is a hardware timer in an RTOS that generates periodic interrupts (ticks) to keep track of system time.

Functions:

- Maintains system time/count (ticks)

- Helps in task scheduling and time slicing

- Supports delay and timeout operations

- Define PWM and give its applications.

Answer:
PWM (Pulse Width Modulation) is a technique used to control the average voltage or power by varying the duty cycle of a digital signal.

Applications:

- Motor speed control

- LED brightness control

- Power regulation in devices

- Signal modulation in communication systems

- Define Serial communication and its advantages.

Answer:
Serial communication is a method of data transmission in which data is sent one bit at a time over a single communication line.

Advantages:

- Requires fewer wires (simple and cost-effective)

- Suitable for long-distance communication

- Less interference/noise compared to parallel communication

- Easy to implement and maintain

- Define an interrupt in embedded systems.

Answer:
An interrupt is a signal that temporarily halts the normal execution of a program and transfers control to a special function called an Interrupt Service Routine (ISR).

Key Point:
After servicing, the CPU resumes the previous task.

- Define a thread.

A thread is the smallest unit of execution within a process, which performs a specific task independently.

- Explain a mutex.

Answer:
A mutex (Mutual Exclusion) is a synchronization mechanism used to protect shared resources by allowing only one task/thread to access the resource at a time.

- Summarize events in RTOS.

Answer:
Events in an RTOS are signals or flags used for communication and synchronization between tasks.

- State the function of NVIC.

Answer:
The NVIC (Nested Vectored Interrupt Controller) is used to manage and control interrupts in a microcontroller.

Functions:

- Handles interrupt prioritization

- Supports nested interrupts (preemption)

- Enables and disables interrupt sources efficiently

- Provides fast interrupt handling using vector table

- Discuss about interrupt latency.

Answer:
Interrupt latency is the time delay between the occurrence of an interrupt and the start of execution of its Interrupt Service Routine (ISR).

- Expand USART and discuss the necessity of USART.

Answer:
USART stands for Universal Synchronous Asynchronous Receiver Transmitter.

Necessity:

- Enables serial communication between microcontroller and external devices

- Supports both synchronous and asynchronous modes

- Used for communication with devices like computers, sensors, and modules

- Provides efficient data transmission with minimal wiring

- Describe a mailbox in RTOS.

Answer:
A mailbox in an RTOS is an inter-task communication mechanism used to send and receive messages (usually a single data item or pointer) between tasks.

- State periodic tasks in RTOS.

Answer:
Periodic tasks in an RTOS are tasks that execute repeatedly at fixed time intervals.

- Define scheduling in RTOS

Answer:
Scheduling in an RTOS is the process of selecting which task should run next based on priority and timing constraints.

4 Marks Questions

- Illustrate with an example how UART communication transfers data between two devices.

Answer:

UART (Universal Asynchronous Receiver Transmitter) is a serial communication protocol that transfers data bit-by-bit without a clock signal, using agreed parameters like baud rate, data bits, parity, and stop bits.

🔹 Working of UART Communication:

- Data is sent in the form of a frame

- Both devices must have the same configuration (baud rate, etc.)

- Transmission is asynchronous (no shared clock)

🔹 UART Frame Format:

A typical frame consists of:

- Start Bit (1 bit, LOW)

- Data Bits (5–9 bits, usually 8 bits)

- Parity Bit (optional, error checking)

- Stop Bit(s) (1 or 2 bits, HIGH)

🔹 Diagram (UART Data Frame):

Idle   Start   Data Bits (LSB → MSB)   Parity   Stop
HIGH    LOW    1 0 1 0 1 0 1 1          0        HIGH
 ────    ─      ─ ─ ─ ─ ─ ─ ─ ─          ─        ────

🔹 Example:

- Device A sends character ‘A’ (ASCII = 65 = 01000001)

- UART transmits:

  - Start bit → 0

  - Data bits → 01000001

  - Stop bit → 1

- Device B receives bits and reconstructs the data back to ‘A’

🔹 Key Points:

- Uses TX (Transmit) and RX (Receive) lines

- Simple and widely used in microcontrollers, GPS, Bluetooth modules

🔹 Conclusion:

UART enables simple, low-cost serial communication between devices using a standard frame structure without clock synchronization.

- Demonstrate with an example how a GPIO pin can control an LED using output mode.

Answer:

A GPIO (General Purpose Input/Output) pin configured in output mode can control an LED by setting the pin HIGH (ON) or LOW (OFF).

🔹 Working:

- GPIO pin is set as OUTPUT

- When the pin outputs HIGH (logic 1) → LED glows

- When the pin outputs LOW (logic 0) → LED turns OFF

- A current-limiting resistor is used to protect the LED

🔹 Example (Pseudo Code):

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, HIGH); // LED ON
delay(1000);

digitalWrite(LED_PIN, LOW);  // LED OFF

🔹 Key Points:

- GPIO acts as a digital switch

- Resistor prevents excess current

- Widely used in embedded systems for indication

🔹 Conclusion:

GPIO pins in output mode provide a simple and effective way to control devices like LEDs by toggling voltage levels.

- Demonstrate queue-based inter-task communication.

Answer:

Queue-based inter-task communication in an RTOS is a mechanism where tasks exchange data through a queue (FIFO – First In First Out) in a safe and synchronized manner.

Working:

- A queue stores data items/messages

- Sender task → places data into the queue (enqueue)

- Receiver task → retrieves data from the queue (dequeue)

- Maintains order of data (FIFO)

- RTOS ensures synchronization (no data corruption)

Diagram (Queue Communication):

(How to draw in exam ✍️):

- Draw: Task A → [QUEUE] → Task B

- Show arrows: enqueue → dequeue

- Label: FIFO

Example:

- Task A (Sensor Task) → sends temperature values to queue

- Task B (Display Task) → reads values from queue and displays

Data flow:

- Task A → Queue (enqueue 25°C, 26°C…)

- Task B → reads in same order → displays values

Key Points:

- Ensures data integrity and synchronization

- Supports communication between tasks and ISR

- Avoids race conditions

Conclusion:

Queues provide a reliable and organized method for inter-task communication, maintaining data order and system stability in RTOS.

- Demonstrate the working principle of timers and counters in embedded systems.

Answer:

Timers and counters in embedded systems are hardware modules used to measure time intervals (timers) and count external/internal events (counters).

Working Principle:

Timer:

- Uses internal clock pulses

- Increments a register at fixed intervals

- When it reaches a preset value (overflow) → generates interrupt or delay

Counter:

- Uses external input pulses

- Increments count for each event (pulse) received

- Used for event counting applications

(How to draw in exam ✍️):

- Draw a register (counter register)

- Show:

  - Internal clock → Timer → Register → Overflow → Interrupt

  - External pulses → Counter → Register

Example:

- Timer: Generate 1-second delay using system clock

- Counter: Count number of people entering a room using sensor pulses

Key Points:

- Timer → time measurement

- Counter → event counting

- Both use register increment mechanism

- Can trigger interrupts on overflow

Conclusion:

Timers and counters are essential for time control and event tracking, making them crucial components in real-time embedded systems.

- Apply the concept of nested interrupts to manage multiple interrupts in an embedded system based on their priority levels.

Answer:

Nested interrupts allow an embedded system to handle multiple interrupts based on priority, where a higher-priority interrupt can preempt a lower-priority ISR.

Working Principle:

- Each interrupt is assigned a priority level

- When an interrupt occurs → CPU executes its ISR

- If a higher-priority interrupt occurs during execution:

  - Current ISR is paused (context saved)

  - Higher-priority ISR is executed first

- After completion → CPU returns to the previous ISR

Diagram (Nested Interrupt Handling):

(How to draw in exam ✍️):

- Show:

  - Main Program → ISR1 (Low Priority)

  - Interrupt occurs → ISR2 (High Priority) interrupts ISR1

  - Return back to ISR1 → Main Program

Example:

- Interrupt 1: Keyboard input (Low priority)

- Interrupt 2: Emergency signal (High priority)

If keyboard ISR is running and emergency interrupt occurs:

- Emergency ISR preempts immediately

- After handling → returns to keyboard ISR

Key Points:

- Uses priority-based interrupt handling

- Requires context saving/restoring

- Supported by controllers like NVIC

- Improves real-time responsiveness

Conclusion:

Nested interrupts ensure critical tasks are handled immediately, improving the efficiency and responsiveness of embedded systems.

- Demonstrate context switching in an RTOS to show how tasks are managed during execution.

Answer:

Context switching in an RTOS is the process of saving the state (context) of the currently running task and restoring the state of another task, allowing multiple tasks to share the CPU efficiently.

Working Principle:

- Each task has its own context (CPU registers, program counter, stack)

- When a task switch is required (e.g., interrupt or scheduler decision):

  - Current task context is saved to its stack

  - Scheduler selects the next task (based on priority)

  - New task context is restored

- CPU resumes execution of the new task

Diagram (Context Switching):

(How to draw in exam ✍️):

- Draw:

  - Task A running → Save Context → Scheduler → Load Context → Task B running

- Show stacks for Task A and Task B

Example:

- Task A: Sensor reading

- Task B: Data processing

If Task B has higher priority:

- RTOS pauses Task A, saves its state

- Switches to Task B

- After completion → resumes Task A from same point

Key Points:

- Enables multitasking

- Uses stack for context storage

- Triggered by interrupts or scheduler

- Ensures efficient CPU utilization

Conclusion:

Context switching is essential in RTOS to manage multiple tasks smoothly, ensuring high-priority tasks execute without delay while preserving task states.

- Demonstrate semaphore-based synchronization in process/task management.

Answer:

A semaphore is a synchronization mechanism used in RTOS to control access to shared resources and coordinate tasks, preventing race conditions.

Types of Semaphores:

- Binary Semaphore (0 or 1) → used for mutual exclusion (like mutex)

- Counting Semaphore → allows multiple instances of a resource

Working Principle:

- Semaphore has a value (S)

- Two atomic operations:

  - Wait (P / take) → decreases S (task waits if S = 0)

  - Signal (V / give) → increases S (wakes waiting task)

Diagram (Semaphore Synchronization):

(How to draw in exam ✍️):

- Draw:

  - Task A → Wait → Critical Section → Signal

  - Task B waits if semaphore = 0

- Label Shared Resource

Example:

- Shared resource: Printer

- Semaphore S = 1

Task A:

- Wait(S) → S = 0 → accesses printer
Task B:

- Wait(S) → blocked (since S = 0)
Task A:

- Signal(S) → S = 1 → Task B proceeds

Key Points:

- Ensures mutual exclusion and synchronization

- Prevents race conditions

- Used in resource sharing and task coordination

Conclusion:

Semaphores provide a safe and efficient way to manage shared resources, ensuring orderly execution of tasks in RTOS.

- Implement GPIO configuration to read input from a push button with an example.

Answer:

A GPIO pin configured in input mode can read the state of a push button as HIGH or LOW, depending on how it is connected.

Working Principle:

- GPIO is set as INPUT

- A pull-up or pull-down resistor ensures a stable logic level

- When button is pressed → logic level changes

- Microcontroller reads the pin using digital input

Circuit Diagram (Push Button with Pull-up):

(How to draw in exam ✍️):

- Draw:

  - Vcc → Resistor → GPIO pin → Push button → GND

- Label:

  - Not pressed → HIGH

  - Pressed → LOW

### Example (Pseudo Code):

pinMode(BUTTON_PIN, INPUT);

if (digitalRead(BUTTON_PIN) == LOW) {
    // Button pressed
}
else {
    // Button not pressed
}

Key Points:

- Prevents floating input using pull-up/down resistor

- Used for user input detection

- Simple and widely used in embedded applications

Conclusion:

GPIO input configuration with a push button allows the system to detect user actions reliably, forming the basis of many interactive embedded systems.

- Demonstrate with an example how interrupts detect a button press event.

Answer:

An interrupt can be used to detect a button press event by triggering an ISR (Interrupt Service Routine) whenever the button state changes, instead of continuously polling.

Working Principle:

- Button is connected to a GPIO pin configured as interrupt input

- Interrupt is set on edge trigger (falling/rising edge)

- When button is pressed → signal change → interrupt occurs

- CPU pauses main program and executes ISR

- After ISR → resumes normal execution

Diagram (Interrupt on Button Press):

(How to draw in exam ✍️):

- Draw:

  - Button → GPIO (Interrupt Pin) → CPU → ISR

- Show:

  - Signal change → Interrupt → ISR execution → Return

Example (Pseudo Code):

void setup() {
    pinMode(BUTTON_PIN, INPUT);
    attachInterrupt(BUTTON_PIN, buttonISR, FALLING);
}

void buttonISR() {
    // Code executed on button press
}

Key Points:

- Faster than polling method

- Uses edge detection (rising/falling)

- Improves CPU efficiency

- Suitable for real-time response

Conclusion:

Interrupts provide an efficient and immediate way to detect button presses, enabling responsive and optimized embedded system behavior.

- Execute task creation using FreeRTOS functions with an example.

Answer:

In FreeRTOS, tasks are created using the function xTaskCreate(), which initializes and schedules tasks for execution.

Function Prototype:

BaseType_t xTaskCreate(
    TaskFunction_t pvTaskCode,
    const char * const pcName,
    uint16_t usStackDepth,
    void *pvParameters,
    UBaseType_t uxPriority,
    TaskHandle_t *pxCreatedTask
);

Working:

- Defines a task function

- Allocates stack memory

- Assigns priority

- Adds task to scheduler (ready state)

(How to draw in exam ✍️):

- Show:

  - Task Function → xTaskCreate() → Ready Queue → Scheduler → Running

Example:

void Task1(void *pvParameters) {
    while(1) {
        // Task code
    }
}

int main() {
    xTaskCreate(Task1, "Task1", 1000, NULL, 1, NULL);
    vTaskStartScheduler(); // Start RTOS scheduler
    while(1);
}

Key Points:

- xTaskCreate() creates and registers a task

- vTaskStartScheduler() starts execution

- Tasks run based on priority scheduling

Conclusion:

Task creation in FreeRTOS enables multitasking, allowing multiple functions to run efficiently and concurrently under RTOS control.

- Execute periodic tasks using a tick timer in RTOS.

Answer:

In an RTOS, periodic tasks are executed at fixed time intervals using the tick timer, which generates regular system ticks (interrupts).

Working Principle:

- RTOS maintains a tick count (incremented every tick interrupt)

- A task uses delay functions like vTaskDelay() or vTaskDelayUntil()

- Task is moved to Blocked state for a specified number of ticks

- After timeout → task returns to Ready state → executed by scheduler

Diagram (Periodic Task Execution):

(How to draw in exam ✍️):

- Draw timeline:

  - Tick → Delay → Blocked → Ready → Running → Repeat

- Show equal intervals between executions

Example (FreeRTOS Code):

void PeriodicTask(void *pvParameters) {
    TickType_t xLastWakeTime = xTaskGetTickCount();

    while(1) {
        // Task code (e.g., read sensor)

        vTaskDelayUntil(&xLastWakeTime, pdMS_TO_TICKS(1000)); // 1 sec periodic
    }
}

Key Points:

- Uses tick timer interrupt

- Ensures fixed periodic execution

- vTaskDelayUntil() is preferred for accurate timing

- Widely used in real-time applications

Conclusion:

Tick timer-based scheduling ensures precise and consistent execution of periodic tasks, making it essential for real-time embedded systems.

11 Marks Questions

- Examine with examples the differences between timers and counters in   embedded systems.

- Evaluate the impact of sampling rate and resolution on the accuracy of an Analog-to-Digital Converter (ADC).

- Analyze with an example how interrupt priorities influence real-time system performance.

- Assess the advantages and limitations of UART communication for serial data transfer in microcontroller systems.

- Analyze different inter-task communication mechanisms in RTOS and their impact on system performance and task synchronization.

- Evaluate synchronization mechanisms such as semaphores and mutexes.

- Differentiate priority inversion in RTOS.

- Evaluate the performance of the FCFS scheduling algorithm in real-time systems with respect to waiting time, response time, and fairness using an example with at least five tasks.

- Analyze the effectiveness of serial communication in embedded systems by examining its advantages, limitations, with illustration using an example like microcontroller to sensor communication or microcontroller to microcontroller communication.

- Evaluate how interrupt priority management using the NVIC improves system responsiveness. Support your evaluation with a suitable example of your choice.

- Analyze with an example how PWM duty cycle affects LED brightness or motor speed.

- Evaluate with examples the use of event-driven programming in embedded system design.

- Analyze the role of context switching in RTOS performance with an example of task scheduling in a multitasking system.

- Judge the effectiveness of preemptive scheduling with an example.

- Analyze inter-task communication mechanisms by identifying their similarities and differences.

- Evaluate the performance of the priority scheduling algorithm in real-time systems considering response time, task criticality, and starvation with a suitable example.

- Analyze the role of interrupts in a real-time embedded application by identifying their types, examining their prioritization strategy for efficient system performance, with support from a suitable real-time example.

- Assess the impact of interrupt latency on the performance of real-time embedded systems.

- Analyze with examples the advantages and limitations of ADC and DAC in embedded systems.

- Evaluate with examples the use of PWM signals for controlling LED brightness and power efficiency.

- Analyze how inter-task communication influences task coordination and system efficiency in RTOS.

- Evaluate the challenges in debugging RTOS-based systems.

- Analyze race conditions in an RTOS using an example such as concurrent access to a shared variable by multiple tasks.

- Recommend appropriate debugging and troubleshooting strategies for an RTOS by evaluating a scenario such as task deadlock.